SVKM’s NMIMS

Mukesh Patel School of Technology Management & Engineering

Department of Electronics and Telecommunication Engineering

Subject: Image and Video Processing 			           Program: B.Tech

Sem: VII    					                                   ACAY: 2020-21

EXPERIMENT NO. 2

Aim:

To write a program in Python to implement spatial resolution (down-sampling and up- sampling) and observe its effect.

Software:  PYTHON.

Prerequisite:

| Sr. No | Concepts |
| --- | --- |
| 1. | Spatial resolution and grey level resolution |

Outcome:

After successful completion of this experiment students will be able to:

- Understand the effect of varying spatial resolution

Theory:

Spatial Resolution:

The term spatial resolution corresponds to the total number of pixels in the given image. If the number of pixels is more, then the resolution of the image is more.

Down-sampling:

In down-sampling technique, number of pixels in the given image is reduced depending on the sampling frequency. Due to this, resolution and size of the image decreases.

Up-sampling:

The number of pixels in the down-sampled image can be increased by using up-sampling interpolation techniques. Up-sampling technique increases the resolution as well as the size of the image.

Some commonly used up-sampling techniques are

- Nearest neighbor interpolation

- Bilinear interpolation

- Cubic interpolation

Algorithm:

Spatial Resolution: Down-sampling

- Read the original image

- Select a down sampling rate say, fs

- New image will consist of pixels as follows:

  - After the pixel in the first column, select every pixel after fs column

  - Similarly, after the first row, select every pixel after fs row

Spatial Resolution: Up-sampling

- Read the above down-sampled image

- Select the up-sampling rate say, fs

- Create a new image of zeros having size of original image

- Fill this new image with pixel values of down-sampled image by skipping rows and columns at rate fs

- The remaining values in the new image can be filled by using neighborhood averaging technique.

Name of the Experiment: To implement spatial resolution variations on the given image

Roll No. C018 Name: Dhruvit Jain

Program: B.Tech ExTC Semester : VII

Date of Performance:17/07/2020 Date of Submission: 17/07/2020

Code:

Down sampling:

from skimage import io

import matplotlib.pyplot as plt

image = io.imread ("C:/Users/dhruv/Desktop/face.png")

io.imshow (image)

img_down = image.copy ()

sh = image.shape

print (sh)

ds = 5

rowd = int(sh[0]/ds) +1

cold = int(sh[1]/ds) +1

img_down = img_down[0:rowd,0:cold]

sh = image.shape

print (sh)

rw=0

cl=0

for r in range (0,sh[0],ds):

cl=0

for c in range (0,sh[1],ds):

img_down [rw][cl]= image[r][c]

cl += 1

rw += 1

sh1 = img_down.shape

print (sh1)

plt.figure()

plt.figure(figsize=(10,10))

plt.subplot(1,2,1)

io.imshow (image)

plt.title ('Original Image')

plt.subplot(1,2,2)

io.imshow (img_down)

plt.title ('Down sampled Image')

Output for down sampling:

ds=5

ds=10

ds=25

Up sampling:

up = 5

img_up = image.copy()

sh = img_down.shape

r=0

c=0

for row in range (0,sh[0]):

c=0

for col in range (0,sh[1]):

img_up[r][c]=img_down[row][col]

img_up [r,c+1:c+up] = 0

c+=up

img_up [r+1 : r+up, :] = 0

r+=up

shu = img_up.shape

#use average method of interpolation

#for columns

for row in range (0,shu[0],up):

for col in range (0,shu[1]-up,up):

temp1 = img_up[row,col]/2

temp2 = img_up[row,col+up]/2

temp = temp1 + temp2

img_up [row,col+1 : col+up] = [int(temp[0]),int(temp[1]),int(temp[2])]  #for RGB

#for rows

for row in range (0,shu[0]-up,up):

for col in range (0,shu[1]):

temp1 = img_up[row,col]/2

temp2 = img_up[row+up,col]/2

temp = temp1 + temp2

img_up [row:row+up,col] = [int(temp[0]),int(temp[1]),int(temp[2])]

plt.figure()

plt.figure(figsize=(10,10))

plt.subplot(1,3,1)

io.imshow (image)

plt.title ('Original Image')

plt.subplot(1,3,2)

io.imshow (img_down)

plt.title ('Down sampled Image')

plt.subplot(1,3,3)

io.imshow (img_up)

plt.title ('Up sampled Image')

Output for up sampling:

Before Average Interpolation:

After Average Interpolation:

Conclusion

- Down Sampling reduces the size of the image in terms of columns and rows.

- As we increase the rate of down sampling, the quality of the image deteriorates.

- For up sampling, if additional rows and columns are replaced by 0, then the quality of image worsens.

- If additional rows and columns are replaced by the average of neighboring pixels, then the quality of image improves.

- Quality of up sampled image can be further improved by using bilinear or cubic interpolation.